| King Saud University | CE 417 |  |
| --- | --- | --- |
| Collage of Engineering | Ch. 4 | Eng. Saad Al-Jadhai |
| Civil Engineering Department | Tutorial No.5 | 2nd semester 1431-1432 |

weight of scraper

100,000

weight of tractor + weight of scraper

80,000 + 100,000

Unit loading and hauling cost ($ / BCY) = total costh   $hproduction BCYh

Trucks and Wagons :

Solve (Example 4-3) if the tractor is a Rear wheel drive ( 40% front Axle and

60% rear Axle )

Solution :  same steps but ..

Maximum usable pull = 44000 (0.6) × 0.45 = 11880 lb

- Effective grade is +10%, Distance one way= 500 ft and  derating factor is 10%

بسبب ضعف كفاءة المحرك فإن الزمن الذي يستغرقه Scraper للتنقل سيزداد

Altitude adjustment :

Travel time = 3.56 * (1 + 0.1) = 3.916 min

Solution:   From fig  travel time = 3.56 min

3) The scraper whose performance curve is shown in figure 4 – 2 is operating at an altitude at which the derating factor is 10%. The scraper gross weight is 150000 lb and operating in grade 3% over a haul road having a rolling resistance factor of 100 lb/ton. What is the maximum speed of the scraper?

Solution:

Effective grade = +3% + (100/20) = 8%

Required pull = 17000 lb

Altitude adjustment = required pull1-derating factor = 170001-0.1 = 18888.89 lb

From fig (4-2)  maximum speed = 6.5 mph