| King Saud University | CE 417 |  |
| --- | --- | --- |
| Collage of Engineering | Ch. 4 | Eng. Saad Al-Jadhai |
| Civil Engineering Department | Tutorial No.6 | 2nd semester 1431-1432 |

600 ft

Starting from 0 + coming to stop

(2000 ft)

coming to stop (600 ft)

Starting from 0 (2000 ft)

1

2

3

Example:

You are given the following data for a scraper job: a. Number of scrapers are seven single engines overhauling; b. tandem pusher will be used; c. the scraper will carry 28 BCY (full load); d. same route will be used for haul and return; e. chain loading method (pusher cycle time is 0.9 min); f. scraper fixed cycle time = 1.3 min; g. efficiency factor is 0.85 and job conditions are average. Sections of the haul route from the cut area to the fill area are as follows:

| Section | Distance (ft) | Grade (%) | Rolling Resistance factor (lb/ton) | Eff. Grade(%) | Max. Speed (mph) | Average speed factor | Average speed (mph) | Travel time (min) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 500 | -3.0 | 100 |  |  |  |  |  |
| 2 | 3,000 | -1.0 | 140 |  |  |  |  |  |
| 3 | 1000 | +1.0 | 180 |  |  |  |  |  |
| 4 | 700 | 0 | 200 |  |  |  |  |  |
| Total travel time |  |  |  |  |  |  |  |  |

Solution:

Effective grades:

Section # 1, effective grade = − 3 + 100/20 = +2

Section # 2, effective grade = − 1 + 140/20 = +6

Section # 3, effective grade = + 1 + 180/20 = +10

Section # 4, effective grade = 0 + 200/20 = +10

Section # 5, effective grade = 0 + 200/20 = +10

Section # 6, effective grade = − 1 + 180/20 = +8

Section # 7, effective grade = +1 + 140/20 = +8

Section # 8, effective grade = +3 + 100/20 = +8

Notes:

- section #3 and section #4 are adjacent ( both are loaded and have same slope)  make it as one section have slope (+10%) and distance (1000+700) = 1700ft

- section #6, section #7 ad section #8 are adjacent ( both are empty and have same slope)  make it as one section have slope  (+8%) and distance (1000+3000+500) = 4500 ft

Maximum speeds: from figure 4 – 2;

Section # 1, loaded: 31 mph

Section # 2, loaded: 14 mph

New section ( # 3,4), loaded: 8 mph

Section # 5, empty: 16 mph

New section (# 6,7,8), empty: 21 mph

● Average speed factor: from table 4 – 3;

Section # 1, starting from zero: 0.65

Section # 2, decreasing max speed from previous section: 1.08

New section ( # 3,4), coming to a stop: by interpolation (distance=1700ft)

(0.86-0.74)/(2000-1000)= x/(1700-1000)  x= 0.084

Average speed factor for New section ( # 3,4) = 0.74+0.084= 0.824

Section # 5, starting from zero: 0.70

New section (# 6,7,8), coming to a stop: (0.93+0.95)/2 = 0.94

● Average speed, mph = maximum speed × average speed factor

| Section | Distance (ft) | Grade (%) | Rolling Resistance factor (lb/ton) | Eff. Grade (%) | Max. Speed (mph) | Average speed factor | Average speed (mph) | Travel time (min) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 500 | -3 | 100 | +2 | 31 | 0.65 | 20.15 | 0.282 |
| 2 | 3000 | -1 | 140 | +6 | 14 | 1.08 | 15.12 | 2.255 |
| 3 | 1000 | +1 | 180 | +10 | 8 | 0.824 | 6.592 | 2.93 |
| 4 | 700 | 0 | 200 | +10 |  |  |  |  |
| 5 | 700 | 0 | 200 | +10 | 16 | 0.7 | 11.2 | 0.71 |
| 6 | 1000 | -1 | 180 | +8 | 21 | 0.94 | 19.74 | 2.59 |
| 7 | 3000 | +1 | 140 | +8 |  |  |  |  |
| 8 | 500 | +3 | 100 | +8 |  |  |  |  |
| Total travel time |  |  |  |  |  |  |  | 8.767 |

● Travel time, min = (distance, ft) / ( 88 * average speed, mph)

Fixed cycle time, table 4 – 7 but it is given 1.3 min

Total cycle time = 1.3+8.767 = 10.07 minutes

Production for one scraper = 28 BCY / 10.07 min = 2.78 BCY/min

= 2.78 * 60 * 0.85  BCY/h = 141.81 BCY/h

Production for seven scrapers = 141.81 * 7 = 992.65  BCY/h

Find the optimum load time for scraper using the following data: cycle time less load time 3.5 minutes, efficiency 50 min/hour, relationships between time of loading and the volume of load are as follows:

| Load time (min) | 0.4 | 0.6 | 0.8 | 1.0 | 1.2 | 1.4 |
| --- | --- | --- | --- | --- | --- | --- |
| Load (BCY) | 16.83 | 20.43 | 22.32 | 23.85 | 24.12 | 24.3 |

| Load time (min) | 0.4 | 0.6 | 0.8 | 1.0 | 1.2 | 1.4 |
| --- | --- | --- | --- | --- | --- | --- |
| Cycle time less load time | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 |
| Cycle time (min) | 3.9 | 4.1 | 4.3 | 4.5 | 4.7 | 4.9 |
| Trips/hour | 15.4 | 14.6 | 14 | 13.3 | 12.76 | 12.2 |
| Load/trip | 16.83 | 20.43 | 22.32 | 23.85 | 24.12 | 24.23 |
| Production (BCY/h) | 216 | 248.56 | 260.4 | 264.33 | 256.5 | 247.08 |

Trips/hour = 60 (min) /3.9 (min/trip) = 15.4 trips

(15.4 trips/h)*(16.83 BCY/trip)*(50min/60min) = 216 BCY/h

Number of scraper served by one pushers