D6.21 For VDD = 1.8 V and using IREF = 50 µA, it is required to design the circuit of Fig. 6.4 to obtain an output current whose nominal value is 50 µA. Find R if Q1 and Q2 are matched with channel lengths of 0.5µm, channel widths of 5 µm, Vt = 0.5 V, and K\n = 250 µA/V2. What is the lowest possible value of Vo? Assuming that for this process technology the Early voltage V\A=20V/µm, find the output resistance of the current source. Also, find the change in output current resulting from a +1-V change in V0.