Q. For a CMOS inverter with matched MOSFET’s having Vt=1V find noise margins if VDD=5V. Find VIL, VIH. (See exercise 4.41).

Exercise4.43

An inverter fabricated in a 1.2µm CMOS technology uses the minimum possible channel lengths (i.e.,Ln = Lp= 1.2 µm). If Wn = 1.8 µm, find the value of Wp that would result in QN and QP being matched. For this technology, k'n = 80 µA/V2, k'p = 27 µA/V2, Vtn = 0.8 V, and VDD = 5V. Also, calculate the value: of the output resistance of the inverter when v0 = V0L.

(See Problem.10.16)

For a CMOS inverster with µnCox=115 µA/V2, WLn=0.375µm/0.25µm, µpCox=30 µA/V2, WLp=1.125µm/0.25µm,VDD=5V, Vtn=|Vtp|=0.5V :

- What is the value of r?

- CalculateVth.

- Plot the VTC (voltage transfer characteristics).

- Repreat for : r=0.5,r=1,r=1.5

For the CMOS with following specifications in Table.1, Find:

- tPHL.

- tPLH

- tP

- Dynamic Power dissipation if it is clocked with 500MHz.

- The delay-power product.

- If inverter is loaded with 0.1pF capacitance, repeat (A,B,C).

| µnCox | 115 µA/V2 | µpCox | 30 µA/V2 |
| --- | --- | --- | --- |
| Vtn=-Vtp | 0.4V | VDD | 2.5V |
| WLn | 0.375µm/0.25µm | WLp | 1.125µm/0.25µm |
| Cgd1 | 0.1125fF | Cgd2 | 0.3375fF |
| Cdb1 | 1fF | Cdb2 | 1fF |
| Cg3 | 0.7875fF | Cg4 | 2.3625fF |
| CW | 0.2fF |  |  |

EE 401: Quiz

Name (Arabic):							Student #

Question  7:	(6 Pts.)

- A CMOS inverter has kn = kp = 330 µA/V2, VDD = 3.3V and Vtn = – Vtp = 0.8 V. - Is the inverter matched? Why? - For this inverter, what are the values of VOH, VOL in the VTC curve? - Find: VIH ,VIL, Vth, NMH, NML, rDSN and rDSP.

(Exersice10.4) For a process technology with L=0.5µm, n=1.5, p=6, give the sizes of all transistors in: (a) A four input NOR and (b) a four input NAND. Also give the relative areas of the two gates.

Question  5(B):	(8 Pts.) (Final 301)

Design a CMOS digital circuit which realizes the function: Z= PQ+RSA+BGHK Specify the sizes of all transistors in terms of n and p so that the worst case delay is equal to that of the basic CMOS inverter.

E D C B A For the pull up circuit shown in the figure, draw the pull down circuit and indicate the transistor ratios to guarantee a worst-case gate delay equal to that of the basic inverter.

| Find the operating regions for Qn and Qp transistors in a Pseudo-NMOS VTC. | region I; region II; region III; region IV. Qn. Qp. |
| --- | --- |

Final 312:

Draw the circuit diagram for the Pseudo-NMOS circuit to realize the following function:

Y=PQ+  R+K.E

EE 401: Quiz

Name (Arabic):							Student #

From the given CMOS Pull-down network:

- Draw the Pull-up network

- Size all transistors in the PUN and PDN to give matching delay to the basic CMOS inverter.