Lab sheet #5

-Qualitative Analysis of Renal Calculi-

-Objectives:

- Identification and qualitative analysis of kidney stones.

-Method:

(1)Test for Uric acid:

1-Put a small amount of the sample.

2-Add 5-7 drops of concentrated nitric acid (Carefully).

3- Heating in a water bath.

(The positive result is yellow to orange color on the inner surface of the test tube).

(2) Test for carbonate:

1- Add 0.5 ml con. hydrochloric acid (2M HCL) to a small portion of the sample.

(Gas bubbles will indicate the presence of carbonate).

(3) Test for oxalate:

1- Heat a part of the sample with 2 ml dilutes sulphuric acid (2M H2SO4) for 1 min.

2-Add 2 drops (one by one) of, potassium permanganate (KMnO4) solution and Mix.

(The decolonization and evolution of bubbles will confirm the presence of oxalate).

(4)Test for phosphates:

1- Dissolve a little of the sample  in about 1.5 ml of concentrated nitric acid (HNO3).

2- Add an equal volume (1.5 ml) of ammonium molybdate solution.

3- Heat to boiling.

(If phosphates are present, a yellow precipitate of ammonium phosphomolybdate is obtained).

(5) Test for calcium:

1- Dissolve small amount of the sample  by heating with 2 ml dilute hydrochloric acid (2M HCL).

2- Add 1 ml ammonium oxalate.

(A white precipitate of calcium oxalate shows the presence of calcium).

(6) Test for magnesium:

1- On a few amount of magnesium, add 1ml of titan followed by 1 ml potassium hydroxide    –to be strongly alkaline-.

(An orange to red color indicates the presence of magnesium).

-Results:

|  | Observation | Type of stone/s |
| --- | --- | --- |
| Uric acid |  |  |
| Carbonate |  |  |
| Oxalate |  |  |
| Phosphates |  |  |
| Calcium |  |  |
| Magnesium |  |  |