Parallel Circuits

6.2 PARALLEL ELEMENTS

| Two elements, branches, or networks are in parallel if they have two points in common. |  |
| --- | --- |

All elements are in parallel

| elements 1 and 2 are in parallel The parallel combination of 1 and 2 is then in series with element 3 |  |
| --- | --- |

| elements 1 and 2 are in series (common point a ) The series combination of 1 and 2 is then in parallel with element 3 |  |
| --- | --- |

6.3 TOTAL CONDUCTANCE AND RESISTANCE

For parallel elements, the total conductance is the sum of the individual conductances.

| Conductance is the inverse of resistance: |  |
| --- | --- |

The total resistance of parallel resistors is always less than the value of the smallest resistor.

| For N equal resistors in parallel |  |
| --- | --- |

For two resistors in parallel:			For three resistors in parallel:

Parallel elements can be interchanged without changing the total resistance or input current.

For parallel resistors, the total resistance will always decrease as additional elements are added in parallel.

For series resistors, the total resistance will always increase as additional elements are added in series.

6.4 PARALLEL CIRCUITS

| Simple Parallel circuit: RT=R1R2R1+R2 and Is=ERT The voltage across parallel elements is the same: V1 = V2 = E I1=V1R1=ER1 and I2=V2R2=ER2 |  |
| --- | --- |

Is=ERT=E∙1RT=E∙1R1+1R2=ER1+ER2=I1+I2

Is=I1+I2

For single-source parallel networks, the source current (Is ) is equal to the sum of the individual branch currents.

| The power dissipated by the resistors and delivered by the source are: |  |
| --- | --- |

6.5 KIRCHHOFF’S CURRENT LAW

| Kirchhoff’s current law (KCL) states that the algebraic sum of the currents entering and leaving an area, system, or junction is zero. The sum of the currents entering an area, system, or junction must equal the sum of the currents leaving the area, system, or junction. |  |
| --- | --- |

| I1 leaving with value: I1 = 22 mA- 17 mA = 5 mA |  |
| --- | --- |

6.6 CURRENT DIVIDER RULE

For two parallel elements of equal value, the current will divide equally.

For parallel elements with different values, the smaller the resistance, the greater the share of input current.

For parallel elements of different values, the current will split with a ratio equal to the inverse of their resistor values.

| I=VRT RT is the total resistance; or V=Ix∙Rx ⟹ I=VRT=Ix∙RxRT ⟹ |  |
| --- | --- |

The current through any parallel branch is equal to the product of the total resistance of the parallel branches and the input current divided by the resistance of the branch.

Current seeks the path of least resistance.

- More current passes through the smaller of two parallel resistors.

- The current entering any number of parallel resistors divides into these resistors as the inverse ratio of their ohmic values.

6.7 VOLTAGE SOURCES IN PARALLEL

Voltage source are placed in parallel only if they have the same voltage rating.

- Primary reason to increase the current rating,	 ⟹	increase power.

| If two batteries of different terminal voltages were placed in parallel, both would be left ineffective or damaged: |  |
| --- | --- |

6.8 OPEN AND SHORT CIRCUITS

| An open circuit can have a potential difference (voltage) across its terminals, but the current is always zero amperes. |  |
| --- | --- |

A short circuit can carry a current of a level determined by the external circuit, but the potential difference (voltage) across its terminals is always zero volts.

| A short circuit is a very low resistance, direct connection between two terminals. |  |
| --- | --- |

| I=ER=10 V2 Ω=5 A | Adding the short across R: RT=2 Ω∥0 Ω=2 Ω∙0 Ω2 Ω+0 Ω=0 Ω I=ERT=10 V0 Ω ⟶∞ A |
| --- | --- |

| I = 0 A; V=22 V | V= 0V; I= 12 mA |
| --- | --- |