# Lab#9: Challenge Spanning Tree Protocol

## Topology Diagram

## Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| S1 | VLAN 99 | 172.17.99.11 | 255.255.255.0 | N/A |
| S2 | VLAN 99 | 172.17.99.12 | 255.255.255.0 | N/A |
| S3 | VLAN 99 | 172.17.99.13 | 255.255.255.0 | N/A |
| PC1 | NIC | 172.17.10.21 | 255.255.255.0 | 172.17.10.12 |
| PC2 | NIC | 172.17.20.22 | 255.255.255.0 | 172.17.20.12 |
| PC3 | NIC | 172.17.30.23 | 255.255.255.0 | 172.17.30.12 |

## Port Assignments – S2

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/1 - 0/5 | 802.1q Trunks (Native VLAN 99) | 172.17.99.0 /24 |
| Fa0/6 - 0/10 | VLAN 30 – Guest(Default) | 172.17.30.0 /24 |
| Fa0/11 - 0/17 | VLAN 10 – Faculty/Staff | 172.17.10.0 /24 |
| Fa0/18 - 0/24 | VLAN 20 - Students | 172.17.20.0 /24 |

## Learning Objectives

Perform basic switch configurations.

Configure the Ethernet interfaces on the host PCs.

Configure VLANs.

Configure spanning tree.

Optimizing STP.

## Introduction

In this activity, you will perform basic switch configurations, configure addressing on PCs, configure VLANs, examine the Spanning Tree Protocol and learn how to optimize it.

Task 1: Perform Basic Switch Configurations

Configure the S1, S2, and S3 switches according to the following guidelines and save all your configurations:

Configure the switch hostname as indicated on the topology.

Disable DNS lookup.

Configure an encrypted privileged EXEC mode password of class.

Configure a password of cisco for console connections.

Configure a password of cisco for vty connections.

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Switch(config)#hostname S1

S1(config)#enable secret class

S1(config)#no ip domain-lookup

S1(config)#line console 0

S1(config-line)#password cisco

S1(config-line)#login

S1(config-line)#line vty 0 15

S1(config-line)#password cisco

S1(config-line)#login

S1(config-line)#end

%SYS-5-CONFIG_I: Configured from console by console

S1#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Your completion results should be 18 percent.  If not, check for missing configuration statements.

Task 2: Configure the Ethernet Interfaces on the Host PCs

From the Desktop tab, use the IP Configuration window to configure the Ethernet interfaces of PC1, PC2, and PC3 with the IP address, subnet mask, and gateway indicated in the addressing table.

Your completion results should be 26 percent.  If not, check for missing configuration steps.

Task 3: Configure VLANs

Step 1. Enable the user ports on S2 in access mode.

Refer to the topology diagram to determine which switch ports on S2 are activated for end-user device access. These three ports will be configured for access mode and enabled with the no shutdown command.

S2(config)#interface fa0/6

S2(config-if)#switchport mode access

S2(config-if)#no shutdown

S2(config-if)#interface fa0/11

S2(config-if)#switchport mode access

S2(config-if)#no shutdown

S2(config-if)#interface fa0/18

S2(config-if)#switchport mode access

S2(config-if)#no shutdown

Step 2. Configure VTP.

Configure VTP on the three switches using the following table. Remember that VTP domain names and passwords are case-sensitive. The default operating mode is server.

| Switch Name | VTP Operating Mode | VTP Domain | VTP Password |
| --- | --- | --- | --- |
| S1 | Server | Lab5 | cisco |
| S2 | Client | Lab5 | cisco |
| S3 | Client | Lab5 | cisco |

S1(config)#vtp mode server

Device mode already VTP SERVER.

S1(config)#vtp domain Lab5

Changing VTP domain name from NULL to Lab5

S1(config)#vtp password cisco

Setting device VLAN database password to cisco

S1(config)#end

S2(config)#vtp mode client

Setting device to VTP CLIENT mode

S2(config)#vtp domain Lab5

Changing VTP domain name from NULL to Lab5

S2(config)#vtp password cisco

Setting device VLAN database password to cisco

S2(config)#end

S3(config)#vtp mode client

Setting device to VTP CLIENT mode

S3(config)#vtp domain Lab5

Changing VTP domain name from NULL to Lab5

S3(config)#vtp password cisco

Setting device VLAN database password to cisco

S3(config)#end

Step 3. Configure Trunk Links and Native VLAN.

Configure trunking ports and native VLAN. For each switch, configure ports Fa0/1 through Fa0/5 as trunking ports. Designate VLAN 99 as the native VLAN for these trunks. When this activity was started, these ports were disabled and must be re-enabled now using the no shutdown command.

Only the commands for the FastEthernet0/1 interface on each switch are shown, but the commands should be applied up to the FastEthernet0/5 interface.

S1(config)#interface fa0/1

S1(config-if)#switchport mode trunk

S1(config-if)#switchport trunk native vlan 99

S1(config-if)#no shutdown

S1(config)#end

S2(config)#interface fa0/1

S2(config-if)#switchport mode trunk

S2(config-if)#switchport trunk native vlan 99

S2(config-if)#no shutdown

S2(config-if)#end

S3(config)#interface fa0/1

S3(config-if#switchport mode trunk

S3(config-if)#switchport trunk native vlan 99

S3(config-if)#no shutdown

S3(config-if-#end

Step 4. Configure the VTP server with VLANs.

VTP allows you to configure VLANs on the VTP server and have those VLANs populated to the VTP clients in the domain. This ensures consistency in the VLAN configuration across the network.

Configure the following VLANS on the VTP server:

| VLAN | VLAN Name |
| --- | --- |
| VLAN 99 | management |
| VLAN 10 | faculty-staff |
| VLAN 20 | students |
| VLAN 30 | guest |

S1(config)#vlan 99

S1(config-vlan)#name management

S1(config)#vlan 10

S1(config-vlan)#name faculty-staff

S1(config)#vlan 20

S1(config-vlan)#name students

S1(config)#vlan 30

S1(config-vlan)#name guest

S1(config-vlan)#end

Step 5. Verify the VLANs.

Use the show vlan brief command on S2 and S3 to verify that all four VLANs have been distributed to the client switches.

S2#show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/4, Fa0/5

Fa0/6, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/11, Fa0/12,Fa0/13

Fa0/14, Fa0/15, Fa0/16,Fa0/17

Fa0/18, Fa0/19, Fa0/20,Fa0/21

Fa0/22, Fa0/23, Fa0/24, Gi0/1

Gi0/2

10   faculty/staff                    active

20   students                         active

30   guest                            active

99   management                       active

S3#show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------------

1    default                          active    Fa0/1, Fa0/2, Fa0/4, Fa0/5

Fa0/6, Fa0/7, Fa0/8, Fa0/9

Fa0/10, Fa0/11, Fa0/12,Fa0/13

Fa0/14, Fa0/15, Fa0/16,Fa0/17

Fa0/18, Fa0/19, Fa0/20,Fa0/21

Fa0/22, Fa0/23, Fa0/24, Gi0/1

Gi0/2

1002 fddi-default                     active

1003 token-ring-default               active

1004 fddinet-default                  active

1005 trnet-default                    active

Step 6. Configure the management interface address on all three switches.

S1(config)#interface vlan99

S1(config-if)#ip address 172.17.99.11 255.255.255.0

S2(config)#interface vlan99

S2(config-if)#ip address 172.17.99.12 255.255.255.0

S3(config)#interface vlan99

S3(config-if)#ip address 172.17.99.13 255.255.255.0

Verify that the switches are correctly configured by pinging between them. From S1, ping the management interface on S2 and S3. From S2, ping the management interface on S3.

Were the pings successful? If not, troubleshoot the switch configurations and try again.

Step 7. Assign switch ports to the VLANs.

Port assignments are listed in the table at the beginning of the activity. However, since Packet Tracer 4.11 does not support the interface range command, only assign the first port from each range.

S2(config)#interface fa0/6

S2(config-if)#switchport access vlan 30

S2(config-if)#interface fa0/11

S2(config-if)#switchport access vlan 10

S2(config-if)#interface fa0/18

S2(config-if)#switchport access vlan 20

S2(config-if)#end

S2#copy running-config startup-config

Destination filename [startup-config]? [enter]

Building configuration...

[OK]

S2#

Your completion results should be 99 percent. If not, check for missing configuration statements.

Task 4: Configure Spanning Tree

Step 1. Examine the default configuration of 802.1D Spanning Tree Protocol (STP).

On each switch, display the spanning tree table with the show spanning-tree command. The output is shown for S1 only. Root selection varies depending on the default BID of each switch. In this activity S3 is currently the root.

S1#show spanning-tree

VLAN0001

Spanning tree enabled protocol ieee

Root ID    Priority    32769

Address     0030.F20D.D6B1

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32769  (priority 32768 sys-id-ext 1)

Address     0050.0F68.146E

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/1            Root FWD 19        128.3    Shr

Fa0/2            Altn BLK 19        128.3    Shr

Fa0/3            Desg FWD 19        128.3    Shr

Fa0/4            Desg FWD 19        128.3    Shr

VLAN0010

Spanning tree enabled protocol ieee

Root ID    Priority    32778

Address     0030.F20D.D6B1

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32778  (priority 32768 sys-id-ext 10)

Address     0050.0F68.146E

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/1            Root FWD 19        128.3    Shr

Fa0/2            Altn BLK 19        128.3    Shr

Fa0/3            Desg FWD 19        128.3    Shr

Fa0/4            Desg FWD 19        128.3    Shr

VLAN0020

Spanning tree enabled protocol ieee

Root ID    Priority    32788

Address     0030.F20D.D6B1

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32788  (priority 32768 sys-id-ext 20)

Address     0050.0F68.146E

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/1            Root FWD 19        128.3    Shr

Fa0/2            Altn BLK 19        128.3    Shr

Fa0/3            Desg FWD 19        128.3    Shr

Fa0/4            Desg FWD 19        128.3    Shr

VLAN0030

Spanning tree enabled protocol ieee

Root ID    Priority    32798

Address     0030.F20D.D6B1

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32798  (priority 32768 sys-id-ext 30)

Address     0050.0F68.146E

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/1            Root FWD 19        128.3    Shr

Fa0/2            Altn BLK 19        128.3    Shr

Fa0/3            Desg FWD 19        128.3    Shr

Fa0/4            Desg FWD 19        128.3    Shr

VLAN0099

Spanning tree enabled protocol ieee

Root ID    Priority    32867

Address     0030.F20D.D6B1

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32867  (priority 32768 sys-id-ext 99)

Address     0050.0F68.146E

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/1            Root FWD 19        128.3    Shr

Fa0/2            Altn BLK 19        128.3    Shr

Fa0/3            Desg FWD 19        128.3    Shr

Fa0/4            Desg FWD 19        128.3    Shr

Note that there are five instances of STP on each switch.

Examine the VLAN 99 spanning tree for all three switches:

S1#show spanning-tree vlan 99

VLAN0099

Spanning tree enabled protocol ieee

Root ID    Priority    32867

Address     0030.F20D.D6B1

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32867  (priority 32966 sys-id-ext 99)

Address     0050.0F68.146E

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/1            Root FWD 19        128.3    Shr

Fa0/2            Altn BLK 19        128.3    Shr

Fa0/3            Desg FWD 19        128.3    Shr

Fa0/4            Desg FWD 19        128.3    Shr

S2#show spanning-tree vlan 99

VLAN0099

Spanning tree enabled protocol ieee

Root ID    Priority    32867

Address     0030.F20D.D6B1

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32867  (priority 32966 sys-id-ext 99)

Address     00E0.F7AE.7258

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/1            Root FWD 19        128.3    Shr

Fa0/2            Altn BLK 19        128.3    Shr

Fa0/3            Altn BLK 19        128.3    Shr

Fa0/4            Altn BLK 19        128.3    Shr

S3#show spanning-tree vlan 99

VLAN0099

Spanning tree enabled protocol ieee

Root ID    Priority    32867

Address     0030.F20D.D6B1

This bridge is the root

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32867  (priority 32966 sys-id-ext 99)

Address     0030.F20D.D6B1

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/1            Desg FWD 19        128.3    Shr

Fa0/2            Desg FWD 19        128.3    Shr

Fa0/3            Desg FWD 19        128.3    Shr

Fa0/4            Desg FWD 19        128.3    Shr

Step 2. Examine the output.

Answer the following questions based on the output.

What is the priority for switches S1, S2, and S3 on VLAN 99?

______________________________________________________________________________

What is the priority for S1 on VLANs 10, 20, 30, and 99?

______________________________________________________________________________

Which ports are blocking VLAN 99 on the root switch?

______________________________________________________________________________

Which ports are blocking VLAN 99 on the non-root switches?

______________________________________________________________________________

How does STP select the root?

______________________________________________________________________________

Since the bridge priorities are all the same, what else does the switch use to determine the root?

______________________________________________________________________________

Task 5: Optimizing STP

Because there is a separate instance of the spanning tree for every active VLAN, a separate root election is conducted for each instance. If the default switch priorities are used in root selection, the same root is elected for every spanning tree, as we have seen. This could lead to an inferior design. Some reasons to control the selection of the root switch include:

The root switch is responsible for generating BPDUs in STP 802.1D and is the focal point for spanning tree control traffic. The root switch must be capable of handling this additional load.

The placement of the root defines the active switched paths in the network. Random placement is likely to lead to suboptimal paths. Ideally the root is in the distribution layer.

Consider the topology used in this activity. Of the six trunks configured, only two are carrying traffic. While this prevents loops, it is a waste of resources. Because the root can be defined on the basis of the VLAN, you can have some ports blocking for one VLAN and forwarding for another. This is demonstrated below.

In this example, it has been determined that the root selection using default values has led to under-utilization of the available switch trunks. Therefore, it is necessary to force another switch to become the root switch for VLAN 99 to impose some load-sharing on the trunks.

In the example output below, the default root switch for all VLANs is S3.

Selection of the root switch is accomplished by changing the spanning-tree priority for the VLAN. The default priority, as you have observed, is 32768 plus the VLAN ID. The lower number indicates a higher priority for root selection. Set the priority for VLAN 99 on S1 to 4096.

S1(config)#spanning-tree vlan 99 priority 4096

S1(config)#exit

Give the switches a little time to recalculate the spanning tree and then check the tree for VLAN 99 on switch S3 (the original VLAN 99 root) and switch S1 (the non-root switch selected to become the new VLAN 99 root).

S3#show spanning-tree vlan 99

VLAN0099

Spanning tree enabled protocol ieee

Root ID    Priority    4195

Address     0050.0F68.146E

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32867  (priority 32966 sys-id-ext 99)

Address     0030.F20D.D6B1

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/4            Desg FWD 19        128.3    Shr

Fa0/1            Root FWD 19        128.3    Shr

Fa0/2            Altn BLK 19        128.3    Shr

Fa0/3            Desg FWD 19        128.3    Shr

S1#show spanning-tree vlan 99

VLAN0099

Spanning tree enabled protocol ieee

Root ID    Priority    4195

Address     0050.0F68.146E

This bridge is the root

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    4195  (priority 4294 sys-id-ext 99)

Address     0050.0F68.146E

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/4            Desg FWD 19        128.3    Shr

Fa0/3            Desg FWD 19        128.3    Shr

Fa0/2            Desg FWD 19        128.3    Shr

Fa0/1            Desg FWD 19        128.3    Shr

Which switch is the root for VLAN 99?

______________________________________________________________________________

Which ports are blocking VLAN 99 traffic on the new root?

______________________________________________________________________________

Which ports are now blocking VLAN 99 traffic on the old root?

______________________________________________________________________________

Compare the S1 VLAN 99 spanning tree above with the S1 VLAN 10 spanning tree.

S1#show spanning-tree vlan 10

VLAN0010

Spanning tree enabled protocol ieee

Root ID    Priority    32778

Address     0030.F20D.D6B1

Hello Time   2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32778  (priority 32788 sys-id-ext 10)

Address     0050.0F68.146E

Aging Time 300

Interface        Role Sts Cost      Prio.Nbr Type

---------------- ---- --- --------- -------- --------------------------------

Fa0/4            Desg FWD 19        128.3    Shr

Fa0/3            Desg FWD 19        128.3    Shr

Fa0/2            Altn BLK 19        128.3    Shr

Fa0/1            Root FWD 19        128.3    Shr

Note that S1 can now use all four ports for VLAN 99 traffic as long as they are not blocked at the other end of the trunk. However, the original spanning tree topology, with one of four S1 ports in blocking mode, is still in place for the four other active VLANs. By configuring groups of VLANs to use different trunks as their primary forwarding path, we retain the redundancy of failover trunks, without having to leaves trunks totally unused.

Your completion results should be 100 percent. If not, use Check Results to find any incomplete components.